# Electronics Documentation

### 1. Electronics Block Diagram

Show signal and power flow

### 2. Components Table

| Component | Quantity | Purpose | Justification |
| --- | --- | --- | --- |
| ESP32 Microcontroller | 1 | Acts as the "brain" to read button presses and calculate motor movements. | It has plenty of connection pins for all the buttons and motors, and it is fast enough to do the CoreXY math instantly. |
| Push Buttons | 6 | To control movement (4 for steering, 1 for up/down, 1 for the claw). | The original analog joystick malfunctioned and was causing drifting issues. Four separate push buttons were used instead to give reliable, exact directional control. |
| DC Motors | 4 | To move the carriage (2), drop the string (1), and grab the prize (1). | They provide enough strength to move the parts quickly. A DC motor was specifically chosen for the claw because the original servo motor was too weak to grip anything. |
| L298N Motor Drivers | 2 | To take high-power electricity from the power supply and send it to the motors. | The ESP32 chip is too weak to power the heavy motors by itself. One driver runs the two moving motors, and the second driver runs the drop and claw motors. |
| Timing Belts & Pulleys (Nylon) | - | To pull the claw carriage around the box. | Required for the CoreXY design. They turn the spinning motion of the motors into smooth, sliding movement for the claw. |