# [Template]

# Test Plan & Results

### 1. Test Plan

| Test | Method | Expected Result |
| --- | --- | --- |
| Electronic Signal Test (Software) | Connect ESP32 to computer and press control panel buttons. | The software accurately reads the 6 buttons and sends the correct power commands to the motor drivers. |
| Mechanical Movement (CoreXY) | Press the directional buttons to drive the X/Y motors. | The pulleys turn the belts, and the claw carriage glides smoothly left, right, forward, and backward. |
| Claw Function Test (Grip) | Press the Drop and Grab buttons to test the claw mechanism. | The string lowers smoothly, and the claw pinches tightly (using the DC motor) to hold a prize. |

### 2. User Testing

- Who were the users? Classmates in the lab.

- What tasks were given? Minimal testing. Because the main belt system failed and the machine could not be played, there was no scope for testing the actual arcade game experience. The testing was limited to handing the detached claw unit to a user. They held the claw in their hand and used the "Grab" button to try and pick up various small objects off a table.

### 3. Results

| Test | Outcome | Pass/Fail |
| --- | --- | --- |
| Electronic Signal Test | The ESP32 "brain" successfully read all buttons and sent power to the motors exactly as programmed. | PASS |
| Mechanical Movement | The X/Y motors spun, but the pulleys slipped on the motor shafts. The belts skipped teeth, and the carriage jammed instantly, making the game unplayable. | FAIL |
| Claw Function Test | When held by hand, the claw motor successfully opened and closed, gripping small objects tightly without dropping them. | PASS (Partial) |

### 4. Observations

- User behavior - The user testing the claw found the button very responsive. They were able to successfully pinch and hold lightweight items (like crumpled paper and small plushies) by holding the claw unit manually over the objects.

- Pain points - It was disappointing that we couldn't test the actual game loop or how the claw behaved when dropping from the string, as the physical machine structure was completely broken.

### 6. Improvements Suggested

- Based on testing - Better Pulley Mounting: Use motors with a flat edge on the shaft (called a D-shaft) and use metal pulleys with strong set-screws so they physically cannot slip when the motor spins.

-   Add Belt Tensioners: The frame needs adjustable screws to pull the belts perfectly tight so they stop skipping over the pulleys.

-   Stronger 3D Printed Mounts: Redesign the motor mounts to be thicker. When the belts pulled, the plastic mounts bent slightly, which caused the system to derail.

- Stepper motors instead of DC motors to be able to have precision and defined movements.

- Neopixel strips to suit the mood.