# Software Documentation

### 1. Software Overview

What does the software do?
This program acts as the main controller for the arcade machine. It sets a 45-second game timer as soon as it powers on. During those 45 seconds, it continuously reads 8 distinct buttons to drive four separate DC motors. It handles the complex "CoreXY" mixing required to drive the X and Y motors together for directional travel, controls a dedicated Z-axis motor for lifting/lowering, and controls a final motor to open and close the claw. Once the timer expires, it completely locks the system to end the game.

Platforms used:
 MicroPython running on an ESP32 microcontroller.

### 2. System Logic

Initialize: The ESP32 assigns its pins, setting eight pins as button inputs (with internal pull-up resistors) and twelve pins as motor outputs (using PWM for speed control). It also starts a master game timer.

Timer Check: The code checks how much time has passed. If 45 seconds have elapsed, it triggers a "TIME UP" state, instantly stops all motors, and traps the code in a dead loop so no more buttons can be pressed.

Read XY Buttons (CoreXY): If the timer is still running, it checks the four directional buttons. If one is pressed, it sends full power to both Motor 1 and Motor 2 in specific forward/backward combinations to move the claw left, right, up, or down.

Read Z-Axis Buttons: It checks the dedicated Z-up and Z-down buttons. If pressed, it drives Motor 3 in the corresponding direction to raise or lower the claw string.

Read Claw Buttons: It checks the claw open and close buttons. If pressed, it drives Motor 4 to pinch or release.

Default Stop State: If you let go of a button, the code automatically sends a "zero speed" command to that specific motor, stopping its movement immediately.

### 3. Flowchart

Visual representation of logic – Initial Idea – A few changes took place i.e. use of buttons instead of joystick, removal of motion sensor and replacement of servo motor with dc motor.

### 4. Code – Final

from machine import Pin, PWM

import time

# BUTTONS

pb_left  = Pin(12, Pin.IN, Pin.PULL_UP)

pb_right = Pin(4, Pin.IN, Pin.PULL_UP)

pb_down  = Pin(5, Pin.IN, Pin.PULL_UP)

pb_up    = Pin(18, Pin.IN, Pin.PULL_UP)

pb_z_up   = Pin(35, Pin.IN, Pin.PULL_UP)

pb_z_down = Pin(16, Pin.IN, Pin.PULL_UP)

pb_claw_open  = Pin(17, Pin.IN, Pin.PULL_UP)

pb_claw_close = Pin(19, Pin.IN, Pin.PULL_UP)

# MOTOR 1 (X)

in1 = Pin(26, Pin.OUT)

in2 = Pin(27, Pin.OUT)

en1 = PWM(Pin(14), freq=1000)

# MOTOR 2 (Y)

in3 = Pin(25, Pin.OUT)

in4 = Pin(33, Pin.OUT)

en2 = PWM(Pin(32), freq=1000)

# MOTOR 3 (Z)

in5 = Pin(21, Pin.OUT)

in6 = Pin(22, Pin.OUT)

en3 = PWM(Pin(23), freq=1000)

# MOTOR 4 (CLAW)

in7 = Pin(13, Pin.OUT)

in8 = Pin(15, Pin.OUT)

en4 = PWM(Pin(27), freq=1000)

# SETTINGS

max_speed = 1023

GAME_TIME = 45   # seconds

start_time = time.time()

def move_motor(in1, in2, en, direction, speed):

if direction == 1:

in1.on()

in2.off()

elif direction == -1:

in1.off()

in2.on()

else:

in1.off()

in2.off()

speed = 0

en.duty(speed)

def stop_all():

move_motor(in1, in2, en1, 0, 0)

move_motor(in3, in4, en2, 0, 0)

move_motor(in5, in6, en3, 0, 0)

move_motor(in7, in8, en4, 0, 0)

while True:

elapsed = time.time() - start_time

# TIME OVER

if elapsed >= GAME_TIME:

print("TIME UP - SYSTEM LOCKED")

stop_all()

# SYSTEM LOCKED

while True:

stop_all()

time.sleep(1)

# XY MOVEMENT

if pb_left.value() == 0:

move_motor(in1, in2, en1, -1, max_speed)

move_motor(in3, in4, en2, -1, max_speed)

print("left")

elif pb_right.value() == 0:

move_motor(in1, in2, en1, 1, max_speed)

move_motor(in3, in4, en2, 1, max_speed)

print("right")

elif pb_down.value() == 0:

move_motor(in1, in2, en1, 1, max_speed)

move_motor(in3, in4, en2, -1, max_speed)

print("down")

elif pb_up.value() == 0:

move_motor(in1, in2, en1, -1, max_speed)

move_motor(in3, in4, en2, 1, max_speed)

print("up")

else:

move_motor(in1, in2, en1, 0, 0)

move_motor(in3, in4, en2, 0, 0)

# Z AXIS

if pb_z_up.value() == 0:

move_motor(in5, in6, en3, 1, max_speed)

print("Z up")

elif pb_z_down.value() == 0:

move_motor(in5, in6, en3, -1, max_speed)

print("Z down")

else:

move_motor(in5, in6, en3, 0, 0)

# CLAW

if pb_claw_open.value() == 0:

move_motor(in7, in8, en4, 1, max_speed)

print("Claw opening")

elif pb_claw_close.value() == 0:

move_motor(in7, in8, en4, -1, max_speed)

print("Claw closing")

else:

move_motor(in7, in8, en4, 0, 0)

time.sleep(0.05)

Codes to test components - 


Claw Code

from machine import Pin, PWM

import time

# L298N Motor Driver Setup

speed_pin = PWM(Pin(19))

speed_pin.freq(1000)

dir_pin_1 = Pin(13, Pin.OUT)

dir_pin_2 = Pin(15, Pin.OUT)

MOTOR_SPEED = 1023

# Main Loop

while True:

# Open claw

print("Opening Claw...")

dir_pin_1.value(1)

dir_pin_2.value(0)

speed_pin.duty(MOTOR_SPEED)

time.sleep(0.8)

dir_pin_1.value(0)

dir_pin_2.value(0)

speed_pin.duty(0)

time.sleep(2)

# Close claw

print("Closing Claw...")

dir_pin_1.value(0)

dir_pin_2.value(1)

speed_pin.duty(MOTOR_SPEED)

time.sleep(0.6)

dir_pin_1.value(0)

dir_pin_2.value(0)

speed_pin.duty(0)

time.sleep(2)

Test code for joystick and motors

from machine import Pin, PWM, ADC

import time

joy_x = ADC(Pin(34))

joy_y = ADC(Pin(35))

joy_x.atten(ADC.ATTN_11DB)

joy_y.atten(ADC.ATTN_11DB)

in1 = Pin(27, Pin.OUT)

in2 = Pin(26, Pin.OUT)

ena = PWM(Pin(14))

ena.freq(1000)

in3 = Pin(25, Pin.OUT)

in4 = Pin(33, Pin.OUT)

enb = PWM(Pin(23))

enb.freq(1000)

MIN_DUTY = 350

MAX_DUTY = 950

time.sleep(3)

center_x = joy_x.read()

center_y = joy_y.read()

while True:

raw_x = joy_x.read()

raw_y = joy_y.read()

x_val = 0.0 if abs(raw_x - center_x) < 200 else (raw_x - center_x) / (float(center_x) if raw_x < center_x else (4095.0 - center_x))

y_val = 0.0 if abs(raw_y - center_y) < 200 else -((raw_y - center_y) / (float(center_y) if raw_y < center_y else (4095.0 - center_y)))

a_mix = max(-1.0, min(1.0, y_val + x_val))

b_mix = max(-1.0, min(1.0, y_val - x_val))

duty_a = 0 if a_mix == 0.0 else int(MIN_DUTY + abs(a_mix) * (MAX_DUTY - MIN_DUTY))

duty_b = 0 if b_mix == 0.0 else int(MIN_DUTY + abs(b_mix) * (MAX_DUTY - MIN_DUTY))

if a_mix > 0:

in1.on(); in2.off()

elif a_mix < 0:

in1.off(); in2.on()

else:

in1.off(); in2.off()

ena.duty(duty_a)

if b_mix > 0:

in3.on(); in4.off()

elif b_mix < 0:

in3.off(); in4.on()

else:

in3.off(); in4.off()

enb.duty(duty_b)

time.sleep(0.1)

Code for joystick – picked from forum about PS2 joysticks.1

from machine import Pin, ADC

from time import sleep

# Joystick axes on ADC1 pins

x_axis = ADC(Pin(34))   # VRx

y_axis = ADC(Pin(35))   # VRy

# Set attenuation for full 0–3.3V range

x_axis.atten(ADC.ATTN_11DB)

y_axis.atten(ADC.ATTN_11DB)

# Joystick button on GPIO32

button = Pin(26, Pin.IN, Pin.PULL_UP)

print("Starting Joystick Check...")

while True:

x_val = x_axis.read()

y_val = y_axis.read()

z_val = button.value()

#print(f"X: {x_val:5} | Y: {y_val:5}")

print(f"X: {x_val:5} | Y: {y_val:5} | Button: {'Pressed' if z_val == 0 else 'Released'}")

sleep(0.2)

DC motor testing codes

from machine import Pin, PWM

import time

#Motor A

in1 = Pin(27, Pin.OUT)

in2 = Pin(26, Pin.OUT)

ena = PWM(Pin(14))

ena.freq(1000)

ena.duty(1023)

# Motor B

in3 = Pin(25, Pin.OUT)

in4 = Pin(33, Pin.OUT)

enb = PWM(Pin(23))

enb.freq(1000)

enb.duty(1023)

# # Motor C

# in5 = Pin(4, Pin.OUT)

# in6 = Pin(5, Pin.OUT)

# enc = PWM(Pin(2))

# enc.freq(1000)

# enc.duty(512)

while True:

# Motor A forward/back

in1.on(); in2.off()

#     time.sleep(3)

#     in1.off(); in2.on()

#     time.sleep(3)

#     in1.off(); in2.off()   # stop motor A

# Motor B forward/back

in3.on(); in4.off()

#     time.sleep(3)

#     in3.off(); in4.on()

#     time.sleep(3)

#     in3.off(); in4.off()   # stop motor B

#     # Motor C forward/back

#     in5.on(); in6.off()

#     time.sleep(3)

#     in5.off(); in6.on()

#     time.sleep(3)

#     in5.off(); in6.off()   # stop motor C

Initial servo motor code



from machine import Pin, PWM

import time

moto =PWM(Pin(32))

moto.freq(50)

while True:

moto.duty(35)

time.sleep(0.2)

moto.duty(110) #0 degree = 18

time.sleep(0.2)