ODT Summative 2

Idea Brief :

A custom-built, electromechanical arcade claw machine that translates simple button presses into smooth 3D physical movement. Powered by an ESP32 microcontroller running MicroPython, the machine reads inputs from a 6-button control panel to drive a "CoreXY" belt system. By securing the DC motors to the stationary frame rather than the moving parts, the claw carriage remains lightweight, allowing it to glide quickly and fluidly across the play area to drop down and grab a prize.


The Frame: A standard boxy arcade structure.

- The Top Mechanics (CoreXY): Smooth metal rods form an X/Y grid zat the top. Two stationary DC motors sit at the back corners, pulling a continuous loop of timing belts.

- The Carriage: A small, lightweight unit that glides on the metal rods, pulled by the belts. It holds a small winch motor (for dropping the string) and a DC-motor-powered claw mechanism.

- The Controls: A separate panel at the front containing 6 push buttons (Forward, Backward, Left, Right, Drop, Grab).

- The Electronics: An ESP32 "brain" and two L298N motor drivers tucked neatly to the side, wiring the buttons to the motors.